IPv4 SUBNETTING:

Unicast: The process of sending a packet from one host to an individual host. IPv4 host addresses are unicast addresses and are in the address range of 0.0.0.0 to 223.255.255.255.

Directed Broadcast: Sending a packet to all hosts on a specific network. Useful when sending to foreign networks. 172.16.4.0/24 destination = 172.16.4.255

Limited Broadcast: Sending a packet to all hosts on the local network. Routers don’t forward these broadcasts. Destination is always IPv4 address 255.255.255.255

Multicast: Sending a packet from one host to a selected group of hosts, possibly to a foreign network. Useful when you want to conserve bandwidth on a network. IPv4 address range of 224.0.0.0 to 239.255.255.255.

Private Address Blocks: 10.0.0.0 to 10.255.255.255 (10.0.0.0/8)

172.16.0.0 to 172.31.255.255 (172.16.0.0/12)

192.168.0.0 to 192.168.255.255 (192.168.0.0/16)

Loopback address - 127.0.0.1 a special address that hosts use to direct traffic to themselves (addresses 127.0.0.0 to 127.255.255.255 are reserved)

Link-Local address - 169.254.0.0 to 169.254.255.255 (169.254.0.0/16) addresses can be automatically assigned to the local host

TEST-NET addresses - 192.0.2.0 to 192.0.2.255 (192.0.2.0/24) set aside for teaching and learning purposes, used in documentation and network examples

Experimental addresses -  240.0.0.0 to 255.255.255.254 are listed as reserved

Binary octets: 128, 64, 32, 16, 8, 4, 2, 1

1011000 = 128 + 32 + 16 = 172

Blah.blah.blah.blah/24 = 24 useable (network) bits, starting from the left, then 8 host bits

ANDing: when lining up the IP address of the host and its subnet mask on top of each other in binary form, the result of the downwards multiplication is the Network Address.

Default Gateway: The sender’s side IP address of the device that routes traffic from the local network to devices on remote networks. In a home or small business environment, the default gateway is often used to connect the local network to the Internet.

Subnet Mask: every bit in the network portion is a 1. Every bit in the host portion is a 0.

Network Address: every bit in the host portion is a 0 when ANDing.

First Host Address: every bit (1 or 0 in octet) in the host portion except for the last bit of the last octet is a 0. Always one more than the Network Address.

Last Host Address: every bit in the host portion except for the last bit of the last octet is a 1. Always one less than the Broadcast Address. When using a prefix of less than 24, add all active bits (all bits with 1’s), except for the last bit of the last octet.

Broadcast Address: every bit of the host portion is a 1.

When determining how many subnets you need, use the following:

No. subnet bits borrowed from hosts	No. of subnets::Subnet Mask Number (SRVM)	Prefix

1	2	::	.128	/25

2	4	::	.192	/26

3	8	::	.224	/27

4	16	::	.240	/28

5	32	::	.248	/29

6	64	::	.252	/30

7	128	::	.254	/31

8	256	::	.255	/32

When determining how many hosts you have, and therefore how many host bits you need to have, use the following:

2^n – 2 = last useable host number, where “n” is the number of host bits left

No. host bits left	no. hosts on each subnet::Subnet Mask Number	 (CIDR)		Prefix

1	2	::	.254	/31

2	4	::	.252	/30

3	8	::	.248	/29

4	16	::	.240	/28

5	32	::	.224	/27

6	64	::	.192	/26

7	128	::	.128	/25

8	256	::	.0	/24

If you need more subnets, take one bit (starting from the right) from the octet to the left for every 256 subnets (or hosts) you need. You are still going down the table when you borrow from right to left, as long as you start from the end of the third octet.

IPv6 SUBNETTING:

About Hexadecimal Format:

0x means Hexadecimal

Hexadecimal is a base sixteen system

Base 16 numbering system uses the numbers 0 to 9 and the letters A to F

Four bits (half of a byte) can be represented with a single hexadecimal value

Hexadecimal	Decimal		Binary

0	0	0000

1	1	0001

2	2	0010

3	3	0011

4	4	0100

5	5	0101

6	6	0110

7	7	0111

8	8	1000

9	9	1001

A	10	1010

B	11	1011

C	12	1100

D	13	1101

E	14	1110

F	15	1111

IPv6 Address Representation:

Each single hexadecimal digit is represented by 4 binary digits. Hexadecimal nibbles are pairs of hexadecimal digits.

Each address is 128 binary bits in length and is written as a string of hexadecimal quartets.

Addresses are not case-sensitive.

First 64 bits are the prefix length and the IPv6 address, last 64 bits are the Interface ID; written like so: IPv6 Address/prefix length : INT ID.

Prefixes can range from 0 to 128 bits in length, but the typical length is 64.

IPv6 Address Notation Compression Rules:

1 – Omit any leading zeros (from left to right), leaving a minimum of one zero if the entire segment was comprised of zeros.

2 – Use a double colon in place of the largest single, contiguous string of one or more 16-bit segments (hextets) consisting of all zeros. Do not use a double colon more than once in the same address. Correct format is like this- :0000:0000:0000: to ::