Packet Tracer - Skills Integration Challenge

Topology

Addressing Table

| Device | Interface | IPv4 Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
|  |  | IPv6 Address/Prefix |  |  |
| R1 | G0/0 | 172.16.10.1 | 255.255.255.192 | N/A |
|  |  | 2001:DB8:CAFE:1::1/64 |  | N/A |
|  | G0/1 | 172.16.10.65 | 255.255.255.192 | N/A |
|  |  | 2001:DB8:CAFE:2::1/64 |  | N/A |
|  | Link-local | FE80::1 |  | N/A |
| S1 | VLAN1 | 172.16.10.62 | 255.255.255.192 | 172.16.10.1 |
| S2 | VLAN1 | 172.16.10.126 | 255.255.255.192 | 172.16.10.65 |
| ManagerA | NIC | 172.16.10.3 | 255.255.255.192 | 172.16.10.1 |
|  |  | 2001:DB8:CAFE:1::3/64 |  |  |
| Accounting.pka | NIC | 172.16.10.2 | 255.255.255.192 | 172.16.10.1 |
|  |  | 2001:DB8:CAFE:1::2/64 |  |  |
| ManagerB | NIC | 172.16.10.67 | 255.255.255.192 | 172.16.10.65 |
|  |  | 2001:DB8:CAFE:2::3/64 |  |  |
| Website.pka | NIC | 172.16.10.66 | 255.255.255.192 | 172.16.10.65 |
|  |  | 2001:DB8:CAFE:2::2/64 |  |  |

Scenario

Your company has won a contract to set up a small network for a restaurant owner. There are two restaurants near each other, and they all share one connection. The equipment and cabling is installed and the network administrator has designed the implementation plan. You job is to implement the rest of the addressing scheme according to the abbreviated Addressing Table and verify connectivity.

Requirements

Complete the Addressing Table documentation.

Configure R1 with IPv4 and IPv6 addressing.

Configure S1 with IPv4 addressing. S2 is already configured.

Configure ManagerA with IPv4 and IPv6 addressing. The rest of the clients are already configured. NOTE: PT cannot configure IPv6 addresses.

Verify connectivity. All clients should be able to ping each other and access the websites on Accounting.pka and Website.pka.

Suggested Scoring Rubric

Packet Tracer scores 80 points. Completing the Addressing Table is worth 20 points.