Packet Tracer - Subnetting Scenario 1

Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0 |  |  |  |
|  | G0/1 |  |  |  |
|  | S0/0/0 |  |  |  |
| R2 | G0/0 |  |  |  |
|  | G0/1 |  |  |  |
|  | S0/0/0 |  |  |  |
| S1 | VLAN 1 |  |  |  |
| S2 | VLAN 1 |  |  |  |
| S3 | VLAN 1 |  |  |  |
| S4 | VLAN 1 |  |  |  |
| PC1 | NIC |  |  |  |
| PC2 | NIC |  |  |  |
| PC3 | NIC |  |  |  |
| PC4 | NIC |  |  |  |

Objectives

Part 1: Design an IP Addressing Scheme

Part 2: Assign IP Addresses to Network Devices and Verify Connectivity

Scenario

In this activity, you are given the network address of 192.168.100.0/24 to subnet and provide the IP addressing for the network shown in the topology. Each LAN in the network requires enough space for, at least, 25 addresses for end devices, the switch and the router. The connection between R1 to R2 will require an IP address for each end of the link.

Design an IP Addressing Scheme

Subnet the 192.168.100.0/24 network into the appropriate number of subnets.

Based on the topology, how many subnets are needed? ___5____

How many bits must be borrowed to support the number of subnets in the topology table? ___3____

How many subnets does this create? ___8____

How many usable hosts does this create per subnet? ___32____

Note: If your answer is less than the 25 hosts required, then you borrowed too many bits.

Calculate the binary value for the first five subnets. The first subnet is already shown.

Net 0: 192 . 168 . 100 .  0   0   0   0   0   0   0   0

Net 1: 192 . 168 . 100 . ___ ___ ___ ___ ___ ___ ___ ___

_________________________________________________________

Net 2: 192 . 168 . 100 . ___ ___ ___ ___ ___ ___ ___ ___

_________________________________________________________

Net 3: 192 . 168 . 100 . ___ ___ ___ ___ ___ ___ ___ ___

_________________________________________________________

Net 4: 192 . 168 . 100 . ___ ___ ___ ___ ___ ___ ___ ___

_________________________________________________________

Calculate the binary and decimal value of the new subnet mask.

11111111.11111111.11111111. ___ ___ ___ ___ ___ ___ ___ ___

____________________________________________________________

255  .   255  .   255  . ______

___________________________________

Fill in the Subnet Table, listing the decimal value of all available subnets, the first and last usable host address, and the broadcast address. Repeat until all addresses are listed.

Note: You may not need to use all rows.

Subnet Table

| Subnet Number | Subnet Address | First Usable Host Address | Last Usable Host Address | Broadcast Address |
| --- | --- | --- | --- | --- |
| 0 |  |  |  |  |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |

Assign the subnets to the network shown in the topology.

Assign Subnet 0 to the LAN connected to the GigabitEthernet 0/0 interface of R1: ___________________

Assign Subnet 1 to the LAN connected to the GigabitEthernet 0/1 interface of R1: ___________________

Assign Subnet 2 to the LAN connected to the GigabitEthernet 0/0 interface of R2: ___________________

Assign Subnet 3 to the LAN connected to the GigabitEthernet 0/1 interface of R2: ___________________

Assign Subnet 4 to the WAN link between R1 to R2: ___________________

Document the addressing scheme.

Fill in the Addressing Table using the following guidelines:

Assign the first usable IP addresses to R1 for the two LAN links and the WAN link.

Assign the first usable IP addresses to R2 for the LANs links. Assign the last usable IP address for the WAN link.

Assign the second usable IP addresses to the switches.

Assign the last usable IP addresses to the hosts.

Assign IP Addresses to Network Devices and Verify Connectivity

Most of the IP addressing is already configured on this network. Implement the following steps to complete the addressing configuration.

Configure IP addressing on R1 LAN interfaces.

Configure IP addressing on S3, including the default gateway.

Configure IP addressing on PC4, including the default gateway.

Verify connectivity.

You can only verify connectivity from R1, S3, and PC4. However, you should be able to ping every IP address listed in the Addressing Table.

Suggested Scoring Rubric

| Activity Section | Question Location | Possible Points | Earned Points |
| --- | --- | --- | --- |
| Part 1: Design an IP Addressing Scheme | Step 1a | 1 |  |
|  | Step 1b | 1 |  |
|  | Step 1c | 1 |  |
|  | Step 1d | 1 |  |
|  | Step 1e | 4 |  |
|  | Step 1f | 2 |  |
| Complete Subnet Table | Step 1g | 10 |  |
| Assign Subnets | Step 2 | 10 |  |
| Document Addressing | Step 3 | 40 |  |
| Part 1 Total |  | 70 |  |
| Packet Tracer Score |  | 30 |  |
| Total Score |  | 100 |  |

Add this note to the submission box when I go to submit the activity.

There seems to be a problem with this page in netacad. The packet tracer activity won’t download; when I click on the link, it just takes me to a weird-looking webpage that says the following: “This XML file does not appear to have any style information associated with it. The document tree is shown below.” So, I have filled out every bit of this document, and I assure you that I know how to apply this information. It’s just like our VLSM Example.