Lab – Subnetting Network Topologies

Objectives

Parts 1 to 5, for each network topology:

Determine the number of subnets.

Design an appropriate addressing scheme.

Assign addresses and subnet mask pairs to device interfaces.

Examine the use of the available network address space and future growth potential.

Background / Scenario

When given a network topology, it is important to be able to determine the number of subnets required. In this lab, several scenario topologies will be provided, along with a base network address and mask. You will subnet the network address and provide an IP addressing scheme that will accommodate the number of subnets displayed in the topology diagram. You must determine the number of bits to borrow, the number of hosts per subnet, and potential for growth as specified by the instructions.

Network Topology A

In Part 1, you have been given the 192.168.10.0/24 network address to subnet, with the following topology. Determine the number of networks needed and then design an appropriate addressing scheme.

Determine the number of subnets in Network Topology A.

How many subnets are there? _____2______

How many bits should you borrow to create the required number of subnets? ____1_____

How many usable host addresses per subnet are in this addressing scheme? ____126_______

What is the new subnet mask in dotted decimal format? ____255.255.255.128_____________________

How many subnets are available for future use? ______0_________

Record the subnet information.

Fill in the following table with the subnet information:

| Subnet Number | Subnet Address | First Usable Host Address | Last Usable Host Address | Broadcast Address |
| --- | --- | --- | --- | --- |
| 0 | 192.168.10.0/25 | 192.168.10.1 | 192.168.10.126 | 192.168.10.127 |
| 1 | 192.168.10.128/25 | 192.168.10.128 | 192.168.10.254 | 192.168.10.255 |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

Network Topology B

The network topology from Part 1 has expanded to accommodate the addition of router R3 and its accompanying network, as illustrated in the following topology. Use the 192.168.10.0/24 network address to provide addresses to the network devices, and then design a new addressing scheme to support the additional network requirement.

Determine the number of subnets in Network Topology B.

How many subnets are there? ____4_______

How many bits should you borrow to create the required number of subnets? ____2_____

How many usable host addresses per subnet are in this addressing scheme? _____62______

What is the new subnet mask in dotted decimal format? _____255.255.255.192____________________

How many subnets are available for future use? ______0_________

Record the subnet information.

Fill in the following table with the subnet information:

| Subnet Number | Subnet Address | First Usable Host Address | Last Usable Host Address | Broadcast Address |
| --- | --- | --- | --- | --- |
| 0 | 192.168.10.0/26 | 192.168.10.1 | 192.168.10.62 | 192.168.10.63 |
| 1 | 192.168.10.64/26 | 192.168.10.65 | 192.168.10.126 | 192.168.10.127 |
| 2 | 192.168.10.128/26 | 192.168.10.129 | 192.168.10.190 | 192.168.10.191 |
| 3 | 192.168.10.192/26 | 192.168.10.193 | 192.168.10.254 | 192.168.10.255 |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |

Network Topology C

The topology has changed again with a new LAN added to R2 and a redundant link between R1 and R3. Use the 192.168.10.0/24 network address to provide addresses to the network devices. Also provide an IP address scheme that will accommodate these additional devices. For this topology, assign a subnet to each network.

Determine the number of subnets in Network Topology C.

How many subnets are there? _____6______

How many bits should you borrow to create the required number of subnets? ____3_____

How many usable host addresses per subnet are in this addressing scheme? _____30______

What is the new subnet mask in dotted decimal format? _____255.255.255.224____________________

How many subnets are available for future use? _______2________

Record the subnet information.

Fill in the following table with the subnet information:

| Subnet Number | Subnet Address | First Usable Host Address | Last Usable Host Address | Broadcast Address |
| --- | --- | --- | --- | --- |
| 0 | 192.168.10.0/27 | 192.168.10.1 | 192.168.10.30 | 192.168.10.31 |
| 1 | 192.168.10.32/27 | 192.168.10.33 | 192.168.10.62 | 192.168.10.63 |
| 2 | 192.168.10.64/27 | 192.168.10.65 | 192.168.10.94 | 192.168.10.95 |
| 3 | 192.168.10.96/27 | 192.168.10.97 | 192.168.10.126 | 192.168.10.127 |
| 4 | 192.168.10.128/27 | 192.168.10.129 | 192.168.10.158 | 192.168.10.159 |
| 5 | 192.168.10.160/27 | 192.168.10.161 | 192.168.10.190 | 192.168.10.191 |
| 6 | 192.168.10.192/27 | 192.168.10.193 | 192.168.10.222 | 192.168.10.223 |
| 7 | 192.168.10.224/27 | 192.168.10.225 | 192.168.10.254 | 192.168.10.255 |
| 8 |  |  |  |  |
| 9 |  |  |  |  |
| 10 |  |  |  |  |

Assign addresses to network devices in the subnets.

Fill in the following table with IP addresses and subnet masks for the router interfaces:

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| R1 | GigabitEthernet 0/1 | 192.168.10.0/27 | 255.255.255.0 |
|  | Serial 0/0/0 | 192.168.10.96/27 | 255.255.255.96 |
|  | Serial 0/0/1 | 192.168.10.128/27 | 255.255.255.128 |
| R2 | GigabitEthernet 0/1 | 192.168.10.32/27 | 255.255.255.32 |
|  | Serial 0/0/0 | 192.168.10.96/27 | 255.255.255.96 |
|  | Serial 0/0/1 | 192.168.10.160/27 | 255.255.255.160 |
| R3 | GigabitEthernet 0/1 | 192.168.10.64/27 | 255.255.255.64 |
|  | Serial 0/0/0 | 192.168.10.128/27 | 255.255.255.128 |
|  | Serial 0/0/1 | 192.168.10.160/27 | 255.255.255.160 |

Fill in the following table with the IP addresses and subnet masks for devices in the LAN as displayed in topology.

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC-A | NIC | 192.168.10.2/27 | 255.255.255.0 | 192.168.10.0/27 |
| PC-B | NIC | 192.168.10.3/27 | 255.255.255.0 | 192.168.10.0/27 |
| S1 | VLAN 1 | 192.168.10.1/27 | 255.255.255.0 | 192.168.10.0/27 |
| PC-C | NIC | 192.168.10.34/27 | 255.255.255.32 | 192.168.10.32/27 |
| PC-D | NIC | 192.168.10.35/27 | 255.255.255.32 | 192.168.10.32/27 |
| S2 | VLAN 1 | 192.168.10.33/27 | 255.255.255.32 | 192.168.10.32/27 |
| PC-E | NIC | 192.168.10.66/27 | 255.255.255.64 | 192.168.10.64/27 |
| PC-F | NIC | 192.168.10.67/27 | 255.255.255.64 | 192.168.10.64/27 |
| S3 | VLAN 1 | 192.168.10.65/27 | 255.255.255.64 | 192.168.10.64/27 |

Network Topology D

The network was modified to accommodate changes in the organization. The 192.168.10.0/24 network address is used to provide the addresses in the network.

Determine the number of subnets in Network Topology D.

How many subnets are there? _____9______

How many bits should you borrow to create the required number of subnets? ____4_____

How many usable host addresses per subnet are in this addressing scheme? _____14______

What is the new subnet mask in dotted decimal format? ____255.255.255.240_____________________

How many subnets are available for future use? ______7_________

Record the subnet information.

Fill in the following table with the subnet information.

| Subnet Number | Subnet Address | First Usable Host Address | Last Usable Host Address | Broadcast Address |
| --- | --- | --- | --- | --- |
| 0 | 192.168.10.0/28 | 192.168.10.1 | 192.168.10.14 | 192.168.10.15 |
| 1 | 192.168.10.16/28 | 192.168.10.17 | 192.168.10.30 | 192.168.10.31 |
| 2 | 192.168.10.32/28 | 192.168.10.33 | 192.168.10.46 | 192.168.10.47 |
| 3 | 192.168.10.48/28 | 192.168.10.49 | 192.168.10.62 | 192.168.10.63 |
| 4 | 192.168.10.64/28 | 192.168.10.65 | 192.168.10.78 | 192.168.10.79 |
| 5 | 192.168.10.80/28 | 192.168.10.81 | 192.168.10.94 | 192.168.10.95 |
| 6 | 192.168.10.96/28 | 192.168.10.97 | 192.168.10.110 | 192.168.10.111 |
| 7 | 192.168.10.112/28 | 192.168.10.113 | 192.168.10.126 | 192.168.10.127 |
| 8 | 192.168.10.128/28 | 192.168.10.129 | 192.168.10.142 | 192.168.10.143 |
| 9 | 192.168.10.144/28 | 192.168.10.145 | 192.168.10.158 | 192.168.10.159 |
| 10 | 192.168.10.160/28 | 192.168.10.161 | 192.168.10.174 | 192.168.10.175 |
| 11 | 192.168.10.176/28 | 192.168.10.177 | 192.168.10.190 | 192.168.10.191 |
| 12 | 192.168.10.192/28 | 192.168.10.193 | 192.168.10.206 | 192.168.10.207 |
| 13 | 192.168.10.208/28 | 192.168.10.209 | 192.168.10.222 | 192.168.10.223 |
| 14 | 192.168.10.224/28 | 192.168.10.225 | 192.168.10.238 | 192.168.10.239 |
| 15 | 192.168.10.240/28 | 192.168.10.241 | 192.168.10.254 | 192.168.10.255 |
| 16 |  |  |  |  |
| 17 |  |  |  |  |

Network Topology E

The organization has a network address of 172.16.128.0/17 to be divided as illustrated in the following topology. You must choose an addressing scheme that can accommodate the number of networks and hosts in the topology.

Determine the number of subnets in Network Topology E.

How many subnets are there? _____9______

How many bits should you borrow to create the required number of subnets? ____4_____

How many usable host addresses per subnet are in this addressing scheme? _____2046______

What is the new subnet mask in dotted decimal format? ____255.255.248.0_____________________

How many subnets are available for future use? ______7_________

Record the subnet information.

Fill in the following table with the subnet information:

| Subnet Number | Subnet Address | First Usable Host Address | Last Usable Host Address | Broadcast Address |
| --- | --- | --- | --- | --- |
| 0 | 172.16.128.0/21 | 172.16.128.1 | 172.16.135.254 | 172.16.135.255 |
| 1 | 172.16.136.0/21 | 172.16.136.1 | 172.16.143.254 | 172.16.143.255 |
| 2 | 172.16.144.0/21 | 172.16.144.1 | 172.16.151.254 | 172.16.151.255 |
| 3 | 172.16.152.0/21 | 172.16.152.1 | 172.16.159.254 | 172.16.159.255 |
| 4 | 172.16.160.0/21 | 172.16.160.1 | 172.16.167.254 | 172.16.167.255 |
| 5 | 172.16.168.0/21 | 172.16.168.1 | 172.16.175.254 | 172.16.175.255 |
| 6 | 172.16.176.0/21 | 172.16.176.1 | 172.16.183.254 | 172.16.183.255 |
| 7 | 172.16.184.0/21 | 172.16.184.1 | 172.16.191.254 | 172.16.191.255 |
| 8 | 172.16.192.0/21 | 172.16.192.1 | 172.16.199.254 | 172.16.199.255 |
| 9 | 172.16.200.0/21 | 172.16.200.1 | 172.16.207.254 | 172.16.207.255 |
| 10 | 172.16.208.0/21 | 172.16.208.1 | 172.16.215.254 | 172.16.215.255 |
| 11 | 172.16.216.0/21 | 172.16.216.1 | 172.16.223.254 | 172.16.223.255 |
| 12 | 172.16.224.0/21 | 172.16.224.1 | 172.16.231.254 | 172.16.231.255 |
| 13 | 172.16.232.0/21 | 172.16.232.1 | 172.16.239.254 | 172.16.239.255 |
| 14 | 172.16.240.0/21 | 172.16.240.1 | 172.16.247.254 | 172.16.247.255 |
| 15 | 172.16.248.0/21 | 172.16.248.1 | 172.16.255.254 | 172.16.255.255 |
| 16 |  |  |  |  |
| 17 |  |  |  |  |

Assign addresses to network devices in the subnets.

Fill in the following table with IP addresses and subnet masks for the router interfaces:

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| R1 | GigabitEthernet 0/0 | 172.16.152.0/21 | 172.16.152.0 |
|  | GigabitEthernet 0/1 | 172.16.160.0/21 | 172.16.160.0 |
|  | Serial 0/0/0 | 172.16.128.0/21 | 172.16.128.0 |
|  | Serial 0/0/1 | 172.16.144.0/21 | 172.16.144.0 |
| R2 | GigabitEthernet 0/0 | 172.16.168.0/21 | 172.16.168.0 |
|  | GigabitEthernet 0/1 | 172.16.176.0/21 | 172.16.176.0 |
|  | Serial 0/0/0 | 172.16.128.0/21 | 172.16.128.0 |
|  | Serial 0/0/1 | 172.16.136.0/21 | 172.16.136.0 |
| R3 | GigabitEthernet 0/0 | 172.16.184.0/21 | 172.16.184.0 |
|  | GigabitEthernet 0/1 | 172.16.192.0/21 | 172.16.192.0 |
|  | Serial 0/0/0 | 172.16.144.0/21 | 172.16.144.0 |
|  | Serial 0/0/1 | 172.16.136.0/21 | 172.16.136.0 |

Reflection

What information is needed when determining an appropriate addressing scheme for a network?

______The number of subnets you need to have in order to accommodate all current hosts on each network. _________________________________________________________________________________

_______________________________________________________________________________________

After the subnets are assigned, will all the host addresses be utilized in each subnet?

____Maybe, but probably not. Each subnet has different needs, and you will likely end up with many host addresses left over. It just depends on how close the quantities of hosts come to the number of available host addresses. ___________________________________________________________________________________

_______________________________________________________________________________________