| 3 |  |  | Top of Form Which protocol is responsible for controlling the size and rate of the HTTP messages exchanged between server and client?. Correct Response; Your Response. HTTP. ARP. TCP. DHCP. TCP is the transport protocol that manages the individual conversations between web servers and web clients. It divides the HTTP messages into smaller pieces (segments) and is responsible for controlling the rate of the messages exchanged between a server and a client. ARP is used to determine the MAC address of a destination host. DHCP is used for automatic IP address assignment and the HTTP protocol governs how a web server and a client can interact.. Bottom of Form |
| --- | --- | --- | --- |
|  | This item references content from the following areas: Introduction to Networks - 3.2.1 Protocols. |
| 4 |  | Top of Form A user is viewing an HTML document located on a web server. What protocol segments the messages and manages the segments in the individual conversation between the web server and the web client?. Correct Response; Your Response. DHCP. TCP. HTTP. ARP. TCP is responsible for segmenting large messages into smaller segments. It also manages the individual conversations between the web server and client. ARP is used to determine the MAC address of the destination. DHCP is used for automatic address assignment and the HTTP protocol governs how a web server and client interact.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 3.2.1 Protocols. |
| --- | --- |
| 7 |  | Top of Form Which IEEE standard enables a wireless NIC to connect to a wireless AP that is made by a different manufacturer?. Correct Response; Your Response. 802.1. 802.11. 802.3. 802.2. The IEEE 802.11 standard defines how wireless LANs operate.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 3.2.3 Standards Organizations. |
| --- | --- |
| 9 |  | Top of Form What is a function of Layer 4 of the OSI model?. Correct Response; Your Response. to specify the packet type to be used by the communications. to apply framing information to the packet, based on the attached media. to represent data to the user, including encoding and dialog control. to describe the ordered and reliable delivery of data between source and destination. Layer 4 of the OSI model (transport layer) describes the general services and functions that can provide the ordered and reliable delivery of data between source and destination hosts. The addition of framing information is described by the data link layer. The type of data packet is specified by the protocol operating at the network layer. The application layer is responsible for representing data to the user including encoding and dialog control.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 3.2.4 Reference Models. |
| --- | --- |
| 10 |  | Top of Form Which statement is true about the TCP/IP and OSI models?​. Correct Response; Your Response. The TCP/IP transport layer and OSI Layer 4 provide similar services and functions.​. The TCP/IP network access layer has similar functions to the OSI network layer.​. The OSI Layer 7 and the TCP/IP application layer provide identical functions.. The first three OSI layers describe general services that are also provided by the TCP/IP internet layer.​. The TCP/IP internet layer provides the same function as the OSI network layer. The transport layer of both the TCP/IP and OSI models provides the same function. The TCP/IP application layer includes the same functions as OSI Layers 5, 6, and 7.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 3.2.4 Reference Models. |
| --- | --- |
| 11 |  | Top of Form What is a benefit of using a layered model for network communications?. Correct Response; Your Response. fostering competition among device and software vendors by enforcing the compatibility of their products. enhancing network transmission performance by defining targets for each layer. avoiding possible incompatibility issues by using a common set of developing tools. simplifying protocol development by limiting every layer to one function. One benefit of using a layered model is to foster competition among vendors because products from different vendors can work together as they perform the same function defined by a specific layer.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 3.2.4 Reference Models. |
| --- | --- |
| 12 |  | Top of Form What is the general term that is used to describe a piece of data at any layer of a networking model?. Correct Response; Your Response. frame. packet. protocol data unit. segment. The term protocol data unit (PDU) is used to describe a piece of data at any layer of a networking model. A packet is the PDU at the network layer. A frame is the data link layer PDU. A segment is the PDU at the transport layer.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 3.3.1 Data Encapsulation. |
| --- | --- |
| 14 |  | Top of Form At which layer of the OSI model would a logical address be encapsulated?. Correct Response; Your Response. physical layer. data link layer. network layer. transport layer. Logical addresses, also known as IP addresses, are encapsulated at the network layer. Physical addresses are encapsulated at the data link layer. Port addresses are encapsulated at the transport layer. No addresses are encapsulated at the physical layer.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 3.3.1 Data Encapsulation. |
| --- | --- |
| 15 |  | Top of Form Which statement accurately describes a TCP/IP encapsulation process when a PC is sending data to the network?. Correct Response; Your Response. Data is sent from the internet layer to the network access layer.. Packets are sent from the network access layer to the transport layer.. Segments are sent from the transport layer to the internet layer.. Frames are sent from the network access layer to the internet layer.. When the data is traveling from the PC to the network, the transport layer sends segments to the internet layer. The internet layer sends packets to the network access layer, which creates frames and then converts the frames to bits. The bits are released to the network media.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 3.3.1 Data Encapsulation. |
| --- | --- |
| 16 |  | Top of Form What statement describes the function of the Address Resolution Protocol?. Correct Response; Your Response. ARP is used to discover the IP address of any host on a different network.. ARP is used to discover the IP address of any host on the local network.. ARP is used to discover the MAC address of any host on a different network.. ARP is used to discover the MAC address of any host on the local network.. When a PC wants to send data on the network, it always knows the IP address of the destination. However, it also needs to discover the MAC address of the destination. ARP is the protocol that is used to discover the MAC address of a host that belongs to the same network.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 3.3.2 Accessing Local Resources. |
| --- | --- |
| 17 |  | Top of Form Which address provides a unique host address for data communications at the internet layer?. Correct Response; Your Response. data-link address. logical address. Layer 2 address. physical address. The Layer 2 address is also called the physical address or the data-link address. The Layer 3 address is also called the logical address.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 3.3.2 Accessing Local Resources. |
| --- | --- |
| 19 |  | Top of Form A user sends an HTTP request to a web server on a remote network. During encapsulation for this request, what information is added to the address field of a frame to indicate the destination?. Correct Response; Your Response. the MAC address of the destination host. the IP address of the destination host. the MAC address of the default gateway. the IP address of the default gateway. A frame is encapsulated with source and destination MAC addresses. The source device will not know the MAC address of the remote host. An ARP request will be sent by the source and will be responded to by the router. The router will respond with the MAC address of its interface, the one which is connected to the same network as the source.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 3.3.3 Accessing Remote Resources. |
| --- | --- |
| 20 |  | Top of Form If the default gateway is configured incorrectly on the host, what is the impact on communications?. Correct Response; Your Response. The host is unable to communicate on the local network.. The host can communicate with other hosts on the local network, but is unable to communicate with hosts on remote networks.. The host can communicate with other hosts on remote networks, but is unable to communicate with hosts on the local network.. There is no impact on communications.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 3.3.3 Accessing Remote Resources. |
| --- | --- |
| 21 |  | Top of Form Question as presented:. The IEEE is a professional organization for those in the electrical engineering and electronics fields. It creates and maintains standards affecting a wide range of industries including telecommunications, and networking. The EIA is an international standards and trade organization for electronics organizations. It is best known for its standards related to electrical wiring, connectors, and the 19-inch racks used to mount networking equipment.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 3.2.3 Standards Organizations. Your response:.. |
| --- | --- |
| 22 |  | Top of Form Open the PT Activity. Perform the tasks in the activity instructions and then answer the question. Based on the configured network, what IP address would PC1 and PC2 use as their default gateway?. Correct Response; Your Response. 192.168.1.2. 10.1.1.1. 172.16.1.1. 192.168.1.1. 192.168.1.10. The default gateway is a router on a LAN segment that is responsible for sending a packet generated by the host to a remote destination. The IP address on the interface connecting the router to the LAN segment is the address used by the host as the default gateway. This can be found by examining the IP addresses configured on the router.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 3.3.3 Accessing Remote Resources. |
| --- | --- |