| 1 |  |  | Top of Form When connectionless protocols are implemented at the lower layers of the OSI model, what are usually used to acknowledge the data receipt and request the retransmission of missing data?. Correct Response; Your Response. connectionless acknowledgements. upper-layer connection-oriented protocols. Network layer IP protocols. Transport layer UDP protocols. Bottom of Form |
| --- | --- | --- | --- |
|  | This item references content from the following areas: Introduction to Networks - 6.1.2 Characteristics of the IP protocol. |
| 2 |  | Top of Form Which IPv4 header field is responsible for defining the priority of the packet?​. Correct Response; Your Response. flow label. flags. differentiated services. traffic class. Differentiated services (DiffServ) is an IPv4 header field that is used to define the priority of each packet. The first 6 bits identify the value that is used by the QoS mechanism, and the last 2 bits identify the value that can be used to avoid packet dropping during network congestion. Traffic class is an IPv6 header field that is equivalent to the IPv4 differentiated services (DiffServ) field. Flow label is also an IPv6 header field that can be used to tell routers and switches to keep the same path for the packet flow to avoid packet reordering. Flags is an IPv4 header field that identifies how the packet is fragmented.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 6.1.3 IPv4 Packet. |
| --- | --- |
| 3 |  | Top of Form Why is NAT not needed in IPv6?​. Correct Response; Your Response. Because IPv6 has integrated security, there is no need to hide the IPv6 addresses of internal networks.​. Any host or user can get a public IPv6 network address because the number of available IPv6 addresses is extremely large.​. The problems that are induced by NAT applications are solved because the IPv6 header improves packet handling by intermediate routers.​. The end-to-end connectivity problems that are caused by NAT are solved because the number of routes increases with the number of nodes that are connected to the Internet.. The large number of public IPv6 addresses eliminates the need for NAT. Sites from the largest enterprises to single households can get public IPv6 network addresses. This avoids some of the NAT-induced application problems that are experienced by applications that require end-to-end connectivity.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 6.1.4 IPv6 Packet. |
| --- | --- |
| 4 |  | Top of Form What is a service provided by the Flow Label field of the IPv6 header?. Correct Response; Your Response. It limits the lifetime of a packet.. It identifies the total length of the IPv6 packet.. It classifies packets for traffic congestion control.. It informs network devices to maintain the same path for real-time application packets.. The Flow Label field is designed to inform devices that an IPv6 packet is part of a flow of packets that should all follow the same path through a network.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 6.1.4 IPv6 Packet. |
| --- | --- |
| 5 |  | Top of Form How do hosts ensure that their packets are directed to the correct network destination?. Correct Response; Your Response. They have to keep their own local routing table that contains a route to the loopback interface, a local network route, and a remote default route.​. They always direct their packets to the default gateway, which will be responsible for the packet delivery.. They search in their own local routing table for a route to the network destination address and pass this information to the default gateway.. They send a query packet to the default gateway asking for the best route.. Hosts must maintain their own local routing table to ensure that network layer packets are directed to the correct destination network. This local table typically contains a route to the loopback interface, a route to the network that the host is connected to, and a local default route, which represents the route that packets must take to reach all remote network addresses.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 6.2.1 How a Host Routes. |
| --- | --- |
| 6 |  | Top of Form Which two commands can be used on a Windows host to display the routing table? (Choose two.). Correct Response; Your Response. netstat -s. route print. show ip route. netstat -r. tracert. On a Windows host, the route print or netstat -r commands can be used to display the host routing table. Both commands generate the same output. On a router, the show ip route command is used to display the routing table. The netstat –s command is used to display per-protocol statistics. The tracert command is used to display the path that a packet travels to its destination.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 6.2.1 How a Host Routes. |
| --- | --- |
| 7 |  | Top of Form During the process of forwarding traffic, what will the router do immediately after matching the destination IP address to a network on a directly connected routing table entry?. Correct Response; Your Response. discard the traffic after consulting the route table. look up the next-hop address for the packet. switch the packet to the directly connected interface. analyze the destination IP address. A router receives a packet on an interface and looks at the destination IP address. It consults its routing table and matches the destination IP address to a routing table entry. The router then discovers that it has to send the packet to the next-hop address or out to a directly connected interface. When the destination address is on a directly connected interface, the packet is switched over to that interface.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 6.2.2 Router Routing Tables. |
| --- | --- |
| 8 |  | Top of Form A technician is configuring a router that is actively running on the network. Suddenly, power to the router is lost. If the technician has not saved the configuration, which two types of information will be lost? (Choose two.). Correct Response; Your Response. Cisco IOS image file. routing table. bootstrap file. ARP cache. startup configuration. Routers contain three types of permanent storage. ROM contains the bootstrap file, flash memory contains the Cisco IOS image file, and NVRAM contains the startup configuration file. These three types of information are not lost when power is lost. Elements that are held in RAM, such as the routing table and ARP cache, are lost when power is turned off.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 6.3.1 Anatomy of a Router. |
| --- | --- |
| 9 |  | Top of Form Which two interfaces will allow access via the VTY lines to configure the router? (Choose two.). Correct Response; Your Response. aux interfaces. LAN interfaces. WAN interfaces. console interfaces. USB interfaces. VTY lines use Telnet or SSH to establish a configuration session to the router. All other interfaces can be used, but not via Telnet or SSH services.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 6.3.1 Anatomy of a Router. |
| --- | --- |
| 10 |  | Top of Form Which two files, if found, are copied into RAM as a router with the default configuration register setting boots up? (Choose two.). Correct Response; Your Response. running configuration. IOS image file. startup configuration. POST diagnostics. The two primary files needed for bootup are the IOS image file and startup configuration, which are copied into RAM to maximize performance. If a router configuration register is set to 0x2102, the router will attempt to load the IOS image from flash memory and the startup configuration file from NVRAM.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 6.3.2 Router Boot-up. |
| --- | --- |
| 13 |  | Top of Form Which three commands are used to set up a password for a person who attaches a cable to a new router so that an initial configuration can be performed? (Choose three.). Correct Response; Your Response. interface fastethernet 0/0. line vty 0 4. line console 0. enable secret cisco. login. password cisco. The three commands needed to password protect the console port are as follows: line console 0 password cisco login Theinterface fastethernet 0/0 command is commonly used to access the configuration mode used to apply specific parameters such as the IP address to the Fa0/0 port. The line vty 0 4 command is used to access the configuration mode for Telnet. The0and 4 parameters specify ports 0 through 4, or a maximum of five simultaneous Telnet connections. The enable secret command is used to apply a password used on the router to access the privileged mode.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 6.4.1 Configure Initial Settings. |
| --- | --- |
| 14 |  | Top of Form Which statement about router interfaces is true?. Correct Response; Your Response. Router LAN interfaces are not activated by default, but router WAN interfaces are.. Once the no shutdown command is given, a router interface is active and operational.. Commands that apply an IP address and subnet mask to an interface are entered in global configuration mode.. A configured and activated router interface must be connected to another device in order to operate.. Interface configuration mode is where the requirements for an active interface are applied. Router LAN and WAN interfaces must be configured in order to be active. In addition to being configured with the no shutdown command, a configured interface must also be connected to another device in order for the physical layer to be active.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 6.4.2 Configure Interfaces. |
| --- | --- |
| 16 |  | Top of Form A technician is manually configuring a computer with the necessary IP parameters to communicate over the corporate network. The computer already has an IP address, a subnet mask, and a DNS server. What else has to be configured for Internet access?. Correct Response; Your Response. the WINS server address. the default gateway address. the MAC address. the domain name of the organization. A computer that connects to any network outside its own broadcast domain needs a default gateway in order to deliver packets. The default gateway address is usually configured by a DHCP server, but if there is not a DHCP server, then it is necessary that the default gateway address be manually configured.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 6.4.3 Configuring the Default Gateway. |
| --- | --- |
| 18 |  | Top of Form Refer to the exhibit. Fill in the blank. A packet leaving PC-1 has to traverse hops to reach PC-4.​. A hop is an intermediary Layer 3 device that a packet has to traverse to reach its destination. In this case, the number of hops a packet has to traverse from PC-1 to PC-4 is three, as there are three routers from source to destination.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 6.1.1 Network Layer in Communication. |
| --- | --- |
| 19 |  | Top of Form Fill in the blank. In a router, is the nonvolatile memory where the diagnostic software, the bootup instructions, and a limited IOS are stored.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 6.3.1 Anatomy of a Router. |
| --- | --- |
| 20 |  | Top of Form Question as presented:. The enable command is entered at the R1> prompt. The login command is entered at the R1(config-line)# prompt. The copy running-config startup-config command is entered at the R1# prompt. The interface fastethernet 0/0 command is entered at the R1(config)# prompt.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 6.4.1 Configure Initial Settings. Your response:.. |
| --- | --- |
| 21 |  | Top of Form Question as presented:. Packets with a destination of 172.17.6.15 are forwarded through Fa0/0. Packets with a destination of 172.17.10.5 are forwarded through Fa1/1. Packets with a destination of 172.17.12.10 are forwarded through Fa1/0. Packets with a destination of 172.17.14.8 are forwarded through Fa0/1. Because network 172.17.8.0 has no entry in the routing table, it will take the gateway of last resort, which means that packets with a destination of 172.17.8.20 are forwarded through Serial0/0/0. Because a gateway of last resort exists, no packets will be dropped.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 6.2.2 Router Routing Tables. Your response:.. |
| --- | --- |
| 22 |  | Top of Form Question as presented:. IPv6 has more QoS and control features than IPv4 has. Fields like Flow Label and Traffic Class in the IPv6 header support those features. Length/Type is not found in Layer 3 headers. It is a Layer 2 field.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 6.1.3 IPv4 Packet - 6.1.4 IPv6 Packet. Continued on bottom.. |
| --- | --- |
| 23 |  | Top of Form Open the PT Activity. Perform the tasks in the activity instructions and then answer the question or complete the task. Does the router have enough RAM and flash memory to support the new IOS?. Correct Response; Your Response. The router has enough RAM and flash memory for the IOS upgrade.. The router has enough RAM, but needs more flash memory for the IOS upgrade.. The router has enough flash memory, but needs more RAM for the IOS upgrade.. The router needs more RAM and more flash memory for the IOS upgrade.. The show version command is used to determine the amount of RAM and flash memory in the router. For RAM look for the line that begins with the words "Cisco CISCO1941/K9 (revision 1.0) with X bytes of memory." For flash memory, review the line that starts "X bytes of ATA System CompactFlash 0 (Read/Write)".. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 6.3.2 Router Boot-up. |
| --- | --- |

MISCELLANEOUS QUESTIONS

Match field names to the IP header where they would be found. Not all options are used.

IPv4 header = Flags, Total Length

IPv6 header = Flow Label, Traffic Class

Refer to the exhibit. Match the packets with their destination IP address to the exiting interfaces on the router. Not all targets are used.

1-6 (top to bottom). 1=6.15; 2=14.8; 3=12.10; 4=10.5; 5=8.20

Match the configuration mode with the command that is available in that mode. Not all options are used.

1-4 (top to bottom). 1=R1>, 2=R1#, 3=R1(config-line)#, 4=R1(config)#