| 1 |  |  | Top of Form What is a socket?. Correct Response; Your Response. the combination of the source and destination IP address and source and destination Ethernet address. the combination of a source IP address and port number or a destination IP address and port number. the combination of the source and destination sequence and acknowledgment numbers. the combination of the source and destination sequence numbers and port numbers. A socket is a combination of the source IP address and source port or the destination IP address and the destination port number.. Bottom of Form |
| --- | --- | --- | --- |
|  | This item references content from the following areas: Introduction to Networks - 7.1.2 Introducing TCP and UDP. |
| 3 |  | Top of Form A host device sends a data packet to a web server via the HTTP protocol. What is used by the transport layer to pass the data stream to the proper application on the server?. Correct Response; Your Response. sequence number. acknowledgment. source port number. destination port number. Destination port numbers are used to pass data streams to their proper applications, while source port numbers are randomly generated and kept track of by the source device. Sequence numbers indicate how to reassemble and reorder received segments. Acknowledgments are used to notify a sending device that a stream of data packets has or has not been received.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 7.1.1 Transportation of Data. |
| --- | --- |
| 4 |  | Top of Form ​What is a beneficial feature of the UDP transport protocol?. Correct Response; Your Response. acknowledgment of received data. fewer delays in transmission. tracking of data segments using sequence numbers. the ability to retransmit lost data. Because its header is much smaller than the TCP header, the UDP protocol provides fewer transmission delays.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 7.1.1 Transportation of Data. |
| --- | --- |
| 5 |  | Top of Form Which scenario describes a function provided by the transport layer?. Correct Response; Your Response. A student is using a classroom VoIP phone to call home. The unique identifier burned into the phone is a transport layer address used to contact another network device on the same network.. A student is playing a short web-based movie with sound. The movie and sound are encoded within the transport layer header.. A student has two web browser windows open in order to access two web sites. The transport layer ensures the correct web page is delivered to the correct browser window.. A corporate worker is accessing a web server located on a corporate network. The transport layer formats the screen so the web page appears properly no matter what device is being used to view the web site.. The source and destination port numbers are used to identify the correct application and window within that application.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 7.1.1 Transportation of Data. |
| --- | --- |
| 7 |  | Top of Form What does a client application select for a TCP or UDP source port number?. Correct Response; Your Response. a random value in the dynamic port range. a random value in the range of the registered ports. a random value in the well-known port range. a predefined value in the dynamic port range. a predefined value in the range of the registered ports. a predefined value in the well-known port range. The client randomly selects an available source port in the range of the registered ports.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 7.1.2 Introducing TCP and UDP. |
| --- | --- |
| 8 |  | Top of Form Compared to UDP, what factor causes additional network overhead for TCP communication?. Correct Response; Your Response. network traffic that is caused by retransmissions. the identification of applications based on destination port numbers. the encapsulation into IP packets. the checksum error detection. All of the extra control tasks that TCP performs, such as ordering segments, keeping track of sequence numbers, asking for retransmissions, and many other tasks, cause the communications to experience delays and overhead.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 7.2.1 TCP Communication. |
| --- | --- |
| 9 |  | Top of Form Which transport layer feature is used to guarantee session establishment?. Correct Response; Your Response. UDP ACK flag. TCP 3-way handshake. UDP sequence number. TCP port number. TCP uses the 3-way handshake. UDP does not use this feature. The 3-way handshake ensures there is connectivity between the source and destination devices before transmission occurs.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 7.2.1 TCP Communication. |
| --- | --- |
| 10 |  | Top of Form Which two flags in the TCP header are used in a TCP three-way handshake to establish connectivity between two network devices? (Choose two.). Correct Response; Your Response. ACK. FIN. PSH. RST. SYN. URG. TCP uses the SYN and ACK flags in order to establish connectivity between two network devices.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 7.2.1 TCP Communication. |
| --- | --- |
| 11 |  | Top of Form Which factor determines TCP window size?. Correct Response; Your Response. the amount of data to be transmitted. the number of services included in the TCP segment. the amount of data the destination can process at one time. the amount of data the source is capable of sending at one time. Window is the number of bytes that the sender will send prior to expecting an acknowledgement from the destination device. The initial window is agreed upon during the session startup via the three-way handshake between source and destination. It is determined by how much data the destination device of a TCP session is able to accept and process at one time.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 7.2.2 Reliability and Flow Control. |
| --- | --- |
| 12 |  | Top of Form During a TCP session, a destination device sends an acknowledgment number to the source device. What does the acknowledgment number represent?. Correct Response; Your Response. the total number of bytes that have been received. one number more than the sequence number. the next byte that the destination expects to receive. the last sequence number that was sent by the source. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 7.2.2 Reliability and Flow Control. |
| --- | --- |
| 13 |  | Top of Form A PC is downloading a large file from a server. The TCP window is 1000 bytes. The server is sending the file using 100-byte segments. How many segments will the server send before it requires an acknowledgment from the PC?. Correct Response; Your Response. 1 segment. 10 segments. 100 segments. 1000 segments. With a window of 1000 bytes, the destination host accepts segments until all 1000 bytes of data have been received. Then the destination host sends an acknowledgment.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 7.2.2 Reliability and Flow Control. |
| --- | --- |
| 14 |  | Top of Form Which two TCP header fields are used to confirm receipt of data?. Correct Response; Your Response. FIN flag. SYN flag. checksum. sequence number. acknowledgment number. Together the TCP sequence number and acknowledgment number fields are used by the receiver to inform the sender of the bytes of data that the receiver has accepted.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 7.2.2 Reliability and Flow Control. |
| --- | --- |
| 15 |  | Top of Form What happens if the first packet of a TFTP transfer is lost?. Correct Response; Your Response. The client will wait indefinitely for the reply.. The TFTP application will retry the request if a reply is not received.. The next-hop router or the default gateway will provide a reply with an error code.. The transport layer will retry the query if a reply is not received.. The TFTP protocol uses UDP for queries, so the TFTP application must implement the reliability, if needed.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 7.2.3 UDP Communication. |
| --- | --- |
| 16 |  | Top of Form What does a client do when it has UDP datagrams to send?. Correct Response; Your Response. It just sends the datagrams.. It queries the server to see if it is ready to receive data.. It sends a simplified three-way handshake to the server.. It sends to the server a segment with the SYN flag set to synchronize the conversation.. When a client has UDP datagrams to send, it just sends the datagrams.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 7.2.3 UDP Communication. |
| --- | --- |
| 17 |  | Top of Form A technician wishes to use TFTP to transfer a large file from a file server to a remote router. Which statement is correct about this scenario?. Correct Response; Your Response. The file is segmented and then reassembled in the correct order by TCP.. The file is segmented and then reassembled in the correct order at the destination, if necessary, by the upper-layer protocol.. The file is not segmented, because UDP is the transport layer protocol that is used by TFTP.. Large files must be sent by FTP not TFTP.. UDP does not reorder the data. If necessary, UDP relies on the upper-layer protocols for reordering of the data.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 7.2.3 UDP Communication. |
| --- | --- |
| 18 |  | Top of Form Fill in the blank. During a TCP session, the field is used by the client to request communication with the server.. The three steps during a TCP three-way handshake are SYN, SYN-ACK, and ACK.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 7.2.1 TCP Communication. |
| --- | --- |
| 19 |  | Top of Form Fill in the blank using a number. A total of messages are exchanged during the TCP session termination process between the client and the server.. To terminate a TCP session, a client sends a segment with the FIN flag set on. The server replies with an ACK flag. Then the server sends a FIN flag to the client. The client finally sends a segment with the ACK flag back to the server. A total of four messages are needed.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 7.2.1 TCP Communication. |
| --- | --- |
| 20 |  | Top of Form Question as presented:. TCP uses 3-way handshaking as part of being able to provide reliable communication and window size to provide data flow control. UDP is a connectionless protocol that is great for video conferencing. Both TCP and UDP have port numbers to distinguish between applications and application windows and a checksum field for error detection.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 7.2.2 Reliability and Flow Control. Your response:.. |
| --- | --- |
| 21 |  | Top of Form Question as presented:. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 7.2.4 TCP or UDP, that is the Question. Your response:.. |
| --- | --- |
| 22 |  | Top of Form Question as presented:. A TCP/IP segment that originated on the PC has 192.168.1.2 as the IP source address. 2578 is the only possible option for the source port number because the PC port number must be in the range of registered ports 1024 to 49151. The destination is the web server, which has the IP address 192.168.2.2, and the destination port number is 80 according to the HTTP protocol standard.. Bottom of Form |

|  | This item references content from the following areas: Introduction to Networks - 7.1.2 Introducing TCP and UDP. Correct response: 1-6 (top to bottom): 2=Source IP address; 3=destination IP address; 5=source port number; 6=destination port number... |
| --- | --- |

MISCELLANEOUS QUESTIONS

Match each application to its connectionless or connection-oriented protocol.

TCP = FTP, Telnet, HTTP

UDP = TFTP, DHCP

Match the characteristic to the protocol category. (Not all options are used.)

TCP = window size, 3-way handshake

UDP = connectionless, best for VoIP

Both UDP and TCP = checksum, port number